|  |  |  |  | D.C MOTOR # DATA SHEET |  |  |  |  |  |  |  | HICO S.O. DM11224 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SHEET OF |  |  |  |  | CUSTOMER : |  |  |  |  |  |  | ITEM NO. : |  |  |  |  |  |
| FOR PROPOSAL |  |  |  |  | JOB NO. : |  |  |  |  |  |  |  |  |  |  |  |  |
| DESCRIPTION : |  |  |  |  | SITE : |  |  |  |  |  |  | QUANTITY : 1 SET |  |  |  |  |  |
| GENERAL DATA |  |  |  |  |  |  | PERFORMANCE DATA |  |  |  |  |  |  |  |  |  |  |
| FRAME NO. |  |  | LAR180LB |  |  |  | OUTPUT |  |  |  |  |  | 45.0 KW ( HP) |  |  |  |  |
| TYPE |  |  |  |  |  |  | SPEED |  |  |  |  |  | 1750 R.P.M |  |  |  |  |
| ENCLOSURE |  |  | DP(IP23S) |  |  |  | ARMATURE |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | ARM. VOLTAGE |  |  |  |  |  | 380 |  |  |  | V |
| COOLING METHOD* |  |  | IC 06 |  |  |  | FULL LOAD CURRENT |  |  |  |  |  | 130.0 |  |  |  | A |
|  |  |  |  |  |  |  | FIELD |  |  |  |  |  |  |  |  |  |  |
| POWER SUPPLY |  |  | NEMA C |  |  |  | POLES |  |  |  |  |  | 4 |  |  |  |  |
|  |  |  |  |  |  |  | WINDING TYPE |  |  |  |  |  | SEP. – SHUNT |  |  |  |  |
| INSULATION CLASS |  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TEMP. RISE AT FULL LOAD |  |  |  |  |  |  | FIELD VOLTAGE |  |  |  |  |  | 180 |  |  |  | V |
|  | RES. METHOD |  | 105 C |  |  |  | FIELD CURRENT |  |  |  |  |  | 5.3 |  |  |  | A |
|  | THERMO. METHOD |  | 85 C |  |  |  | FIELD RES.(AT25C) |  |  |  |  |  | 25.66 |  |  |  | Ω |
| LOCATION |  |  | INDOOR |  |  |  | EFFICIENCY |  |  |  |  |  |  |  |  |  |  |
| ALTITUDE |  |  | LESS THAN 1000M |  |  |  |  |  | AT 1/2 LOAD |  |  |  | - |  |  |  | % |
| HUMIDITY |  |  | LESS THAN 80% |  |  |  |  |  | AT 3/4 LOAD |  |  |  | - |  |  |  | % |
| AMBIENT TEMP.(MAX.) |  |  | 40 C |  |  |  |  |  | AT FULL LOAD |  |  |  | 90 |  |  |  | % |
| RATING |  |  | CONTINUOUS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ROTATION |  |  | C.W |  |  |  | FULL LOAD TORQUE |  |  |  |  |  | 25.04 |  |  |  | Kg.m |
| MOUNTING |  |  | B3 (HORIZONTAL) |  |  |  | MOTOR GD2 |  |  |  |  |  | 1.6 |  |  |  | Kg.m2 |
| BEARING TYPE |  |  | ANTI FRICTION |  |  |  | CARBON BRUSH |  |  |  |  |  |  |  |  |  |  |
| FRONT / REAR BRG. NO. |  |  | 6313 / 6313 |  |  |  | BRUSH MATERIAL |  |  |  |  |  | EG332 |  |  |  |  |
| BRG. LUBRICATION |  |  | GREASE |  |  |  | BRUSH SIZE |  |  |  |  |  | 10x32x40 |  |  |  |  |
| SERVICE FACTOR |  |  | 1.00 |  |  |  | MOTOR APPROX. WEIGHT |  |  |  |  |  | 385 |  |  |  | Kg |
| DRIVE |  |  | DIRECT COUPLED |  |  |  | PAINTING |  |  |  |  |  | 8B 4.5/3.25 |  |  | (URETOP) |  |
| SHAFT |  |  |  |  |  |  | SUBMITTAL DRAWINGS |  |  |  |  |  |  |  |  |  |  |
|  | EXTENSION |  | SINGLE |  |  |  | OUTLINE DIMENSION |  |  |  |  |  | DDMK10825H01 |  |  |  |  |
|  | VERTICAL |  |  |  |  |  | CONDUIT BOX & COVER |  |  |  |  |  | 1DO 00383 |  |  |  |  |
|  | EXTERNAL THRUST |  |  |  |  |  | TERMINAL DIAGRAM |  |  |  |  |  | 1DO 00302 |  |  |  |  |
| MOTOR MOUNTED BLOWER |  |  |  |  |  |  | . |  |  |  |  |  |  |  |  |  |  |
|  | OUTPUT |  | 2.0HPX2P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | VOLTAGE |  | 220/380 |  |  | V | <NOTE> EXCEPT FOR STATEMENTS SPECFIED ON THIS |  |  |  |  |  |  |  |  |  |  |
|  | PHASE/FREQUNCY |  | 3 PHASE 60Hz |  |  |  | SHEET,ANYTHING ELSE SHALL BE IN ACCORDANCE |  |  |  |  |  |  |  |  |  |  |
| MOTOR MOUNTED TACHO-GENERATOR |  |  |  |  |  |  | WITH HYOSUNG STANDARS. |  |  |  |  |  |  |  |  |  |  |
|  | TYPE |  | 104-8011-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | OUTPUT |  | 30V/1000RPM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | MAKER |  | SANYO |  |  |  | <REMARKS> |  |  |  |  |  |  |  |  |  |  |
| TERMINAL BOX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MAIN |  |  | HYOSUNG STD. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AUX. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| APPLICATION STANDARDS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IEC 34 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <NOTE> THE DATA OF THE DARK LINE MUST BE CHECKED FOR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPARE MOTOR. IF THE CONTROL PANEL DOES NOT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SATIFY THE REQUIRMENTS, IT MUST BE ADJUSTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | REV. NO |  |  | DATE | PREPD BY |  |  | CHKD BY | REV. NO |  |  |
| *IC06 : MOTOR MOUNTED FAN AND FREE CIRCULATION IC17 : PIPE VENTILATED INLET DUCT AND FREE CIRCULATION IC37 : PIPE VENTILATED INLET AND OUTLET DUCTS IC0041 : TOTALLY ENCLOSED IC0641 : TOTALLY ENCLOSED,FAN COOLED ** ROTATION : SHAFT EXTENTION VIEW |  |  |  |  |  |  | 0 |  |  | 2011.12.01 | Y.H.JANG |  |  | K.S.LEE | K.S.LEE |  |  |